2PP Setup API Platform — Requirements

Milestones 1 & 2  ·  derived from the approved architecture and Mushegh's milestone feedback (24 Jul 2026)

Status: Draft v1 for team review   ·   Owner: Narek   ·   Reviewers: Mushegh, Tatevik, Tetiana, Aram

Implementation status: a first working version of the Milestone-1 platform ('labgate') exists on branch feature/labgate-platform — 68 automated tests, fully runnable in simulation mode without hardware. The open questions in §7 gate the remaining work (camera/WL adapters, rig bring-up, chat platform integration).

# 1.  Context

The architecture proposal was reviewed and the general plan approved. Two milestones are green-lit:

Milestone 1 — The 2PP Setup API Platform (layers L0–L3): one unified API platform integrating the laser, 3D stage, camera, and white-light (WL) source on the custom 2PP setup. All four units are already automated individually; documentation and SDKs live on the setup computer. Contacts: Tatevik Sarukhanyan, Tetiana Orlova.

Milestone 2 — Proof-of-principle AI interface & validation loop: connect the platform to chat.photonics.ai (built by Aram), where an Optical Microfabrication Specialist agent (user-facing) and an Experimentalist agent (background) plan, cross-validate, obtain user approval, execute via our API, and analyze results.

Overriding design goal: the platform must be integratable into any environment — any UI, any chat platform, any agent framework, any scripting language. chat.photonics.ai is the first client, never a dependency.

# 2.  Scope

In scope (M1): device adapters for the four units; capability registry; declarative Experiment Specification; validation engine; plan lifecycle with approval; execution engine (owns synchronization); results/artifacts; audit log; auth with roles; simulation mode; REST API + OpenAPI spec.

In scope (M2): integration contract + guide for chat.photonics.ai; example client; support for the two-agent workflow; result transfer back to the platform agents.

Out of scope (for now): structure-design assistance (roadmap Phase F); replacing the hardware sync mechanism (it gets encapsulated, improved later); building any UI of our own beyond API docs.

# 3.  Functional requirements — Milestone 1

| ID | Requirement |
| --- | --- |
| F1 | Expose four device classes behind one API: motion stage, laser, camera, white-light source. |
| F2 | Each device adapter declares its capabilities (operations with typed parameters, units, hard bounds, pre/postconditions) in machine-readable form; queryable via GET /capabilities so any AI client can ground itself without prior knowledge. |
| F3 | Experiments are described by a declarative Experiment Specification (JSON/YAML, schema-versioned): an ordered list of high-level operations. Specs are stored, versioned, and re-runnable without any AI. |
| F4 | Server-enforced plan lifecycle: draft → validated → approved → queued → running → completed / failed / aborted, with endpoints to create, validate, dry-run, approve, execute, monitor, abort, and fetch results. Invalid transitions rejected server-side. |
| F5 | A deterministic validation engine checks every spec against parameter bounds, stage travel range, exposure/power policy, mutual device constraints, and current device state; it produces a human-readable report feeding the approval step and the agents. |
| F6 | Dry-run simulates a validated plan without hardware: estimated duration, toolpath envelope, exposure summary. |
| F7 | The execution engine is the only component that commands hardware. It expands specs into device commands, owns stage↔laser synchronization, holds per-device single-writer locks, and on any fault drives devices to safe state (laser off first). |
| F8 | Runs produce retrievable results: telemetry/event log and camera artifacts (images), via GET /plans/{id}/results. |
| F9 | An append-only audit log records every request, validation outcome, approval (with approver identity), execution event, and fault. |
| F10 | Token-based authentication; roles operator (propose/execute), approver (approve), admin. The platform enforces proposer ≠ approver server-side. |
| F11 | Simulation mode: the entire platform runs with simulated adapters (no hardware), enabling development off the rig, CI testing, and safe agent-integration testing for M2. |
| F12 | Device health/state queryable (GET /devices): connection status, last-known positions/settings. |

# 4.  Non-functional requirements (integratability first)

| ID | Requirement |
| --- | --- |
| N1 | UI-agnostic contract: pure HTTP/JSON REST with an auto-generated OpenAPI 3 document. No assumption about the client (chat platform, script, GUI, agent framework). |
| N2 | MCP-ready: the capability registry and endpoints map cleanly onto Model Context Protocol tools so a thin MCP wrapper can be added later without touching the core. |
| N3 | Extensible devices: adding a device = writing one adapter implementing the adapter interface + registering it. Zero changes to core code. |
| N4 | Deployable on the setup computer: Python, minimal dependency footprint, single-process service; no cloud dependency to operate locally. |
| N5 | The API is the only path to hardware. Agents, UIs, and scripts never import drivers directly. |
| N6 | Status via polling first (simple, universal); streaming (SSE/WebSocket) optional later. |
| N7 | All timestamps, units, and coordinates explicit in the schema (µm, mm/s, mW, ISO-8601) — agents must never guess units. |

# 5.  Milestone 2 — mapping the two-agent workflow onto the API

The platform stays agent-agnostic; the approved workflow maps onto plain API calls:

| Workflow step (per Mushegh) | API interaction |
| --- | --- |
| Specialist + user discuss; Experimentalist formulates plan | GET /capabilities, GET /devices for grounding; POST /plans with the spec |
| Experimentalist sends plan back to Specialist for validation | read the plan's deterministic validation report + POST /plans/{id}/dry-run; agents add domain judgment on top |
| Specialist presents plan to user; user confirms in chat | POST /plans/{id}/approve — must carry an identity distinct from the proposer (see Q-A3) |
| Experimentalist executes | POST /plans/{id}/execute; poll GET /plans/{id} |
| Results transferred back; Specialist analyzes | GET /plans/{id}/results (telemetry + images) |

Note: the agents' internal cross-validation is a quality layer above the trust boundary. It complements — never replaces — the platform's deterministic validation (F5) and role-enforced approval (F10).

# 6.  Constraints & assumptions

Stage: Standa 8MTL120XY via ACS SPiiPlus (SPiiPlusPython from local ACS ADK wheel; Ethernet TCP 10.0.0.100:701; ±25 mm; 1 µm).

Laser: Phoebe PH2 via HTTP REST (192.168.244.10/phoebe/v0/…).

Camera & WL source: automated per the feedback; SDKs on the setup computer — models/interfaces unknown until we get the docs (Q-H1/Q-H2). Adapters ship as simulated + stub-real until then.

Current stage↔laser sync is timing-calibrated (sleep-based); it is encapsulated behind a synchronizer interface in M1, improved later.

Independent hardware interlocks exist on the rig — the software safety layer sits above them.

Rig computer is Windows with Python 3.12 (.venv312 holds the vendor wheel); 3.13 is allowed for simulation-mode development off the rig.

# 7.  Questions for the team

As requested, all open questions gathered in one place. The Answer column is intentionally blank — to be filled during the clarification discussion.

## 7.1  Hardware / current automation — Tatevik & Tetiana

| ID | Question | Answer |
| --- | --- | --- |
| Q-H1 | Camera — exact model, SDK (vendor lib? GenICam? OpenCV-compatible?), and the current automation script. Which capture parameters matter (exposure, gain, ROI)? |  |
| Q-H2 | WL source — model and control interface (serial? USB? vendor DLL?), controllable parameters (on/off? intensity?), and the current script. |  |
| Q-H3 | Mutual-constraint rules we must encode? (e.g., WL off during 2PP exposure; camera capture requires WL on; shutter states.) |  |
| Q-H4 | Definitive safe-state per device (laser: off vs. shutter closed; stage: stop vs. home; WL: off)? |  |
| Q-H5 | Setup computer: OS + version, Python available, can we run a persistent local service on it? |  |

## 7.2  Safety & policy — Mushegh

| ID | Question | Answer |
| --- | --- | --- |
| Q-S1 | Who signs off the hard bounds table (max laser power, max stage speed, exposure limits) that the validation engine enforces? |  |
| Q-S2 | Confirm the interlock inventory on this specific setup (e-stop, shutter/key, enclosure). |  |

## 7.3  Approval & identity — Mushegh & Aram

| ID | Question | Answer |
| --- | --- | --- |
| Q-A1 | How does chat.photonics.ai authenticate users, and can it pass the user's identity through to our API (needed for audit + proposer ≠ approver)? |  |
| Q-A2 | Who are the designated approvers? Is the chatting user ever also the approver, or must a qualified person approve (the architecture recommends proposer ≠ approver)? |  |
| Q-A3 | Is a chat reply ('I confirm…') acceptable as the approval UI, provided the platform still enforces an authenticated approve call with a distinct approver identity underneath? |  |

## 7.4  Platform integration — Aram

| ID | Question | Answer |
| --- | --- | --- |
| Q-P1 | Where does chat.photonics.ai run (cloud/on-prem)? What network path exists to the setup computer (VPN? tunnel? same LAN?) — do we need to expose the API beyond the lab network? |  |
| Q-P2 | How do the Specialist/Experimentalist agents call external tools — OpenAPI/function calling, MCP, or custom HTTP? (The platform provides OpenAPI now, MCP wrapper on request.) |  |
| Q-P3 | How should large artifacts (images) be transferred — inline base64, download URLs, or a shared store? |  |
| Q-P4 | Who authors/owns the two agents' prompts and tool definitions — the chat-platform side, or do we supply a 'tool manifest' they load? |  |

## 7.5  Process — Mushegh

| ID | Question | Answer |
| --- | --- | --- |
| Q-M1 | Definition of done for M1: is simulation-mode demo + stage/laser live on the rig sufficient, with camera/WL following once SDK docs are in hand? |  |
| Q-M2 | For M2's proof of principle, which experiment class should the demo run — parameter-sweep arrays (recommended: bounded, well-understood) or STL prints? |  |

# 8.  Traceability to the approved architecture

| Architecture layer (proposal §4) | Realization |
| --- | --- |
| L0–L1 Rig + interlocks / device firmware | Existing hardware (interlocks confirmed present) |
| L2 Device abstraction layer | Platform device adapters + capability registry (F1–F2) |
| L3 API-Gate / orchestration core | Validation, lifecycle, approval, executor, audit, auth (F4–F10, F12) |
| L4 Experiment Specification | Declarative spec schema (F3) |
| L5–L6 AI planning + conversational UI | chat.photonics.ai agents + chat (Aram's side, Milestone 2) |

The four design principles remain binding on every implementation decision: the AI proposes / the deterministic layer disposes; the AI's output is untrusted input; the model authors intent, not trajectories; value before AI.